Endangered Species Act

Stormwater Design Checklist

For Western Washington

Table of Contents

Endangered Species Act Stormwater Design Checklist	1

Purpose and Use of the Checklist	1

Endangered Species Act Stormwater Design Checklist Form	3

Endangered Species Act 
Stormwater Design Checklist

## Purpose and Use of the Checklist

The Stormwater Design Checklist assists project designers in providing pertinent information about a project’s stormwater treatment facilities to biologists responsible for preparing biological assessments required for consultation under Section 7 of the Endangered Species Act.  The use of this checklist is necessary to aid in developing biological assessments and promoting consistency in the content provided in the agency’s biological assessments.

It is possible that the specific conditions of some projects may warrant modifying or adding certain checklist items.  However, to maintain consistency in the type and amount of information collected and submitted for the environmental permitting process, the checklist should be modified only if necessary.

# Endangered Species Act
Stormwater Design Checklist

Project Name:_____________________________________________________________

Project Location:__________________________________________________________

Part 1

General Project Information

1.	Will work occur outside existing pavement or gravel shoulders?    Yes    No

If yes, describe the nature and extent of the work:

_______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

2.	Is off-site stormwater being treated/controlled by WSDOT stormwater facilities prior to initiation of the project?    Yes    No

If yes, will this stormwater continue to be treated/controlled to the same level?   Yes   No

If off-site stormwater will not continue to be treated/controlled to the same level, explain why not:

_______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Existing Stormwater Facilities (Pre-project)

3.	How many TDAs exist within in the project area, how many outfalls or discharge point(s) are located in each TDA, and what are the receiving waterbodies?

| Existing TDA Number | Number of Discharge Points/Outfalls | Receiving Waterbody |
| --- | --- | --- |

4.	For each existing TDA/outfall (subdivide TDAs if there are multiple outfalls), identify total TDA area, area of impervious surface, area of impervious surface receiving runoff treatment, the runoff BMP type(s),  area of impervious surface not receiving runoff treatment, area of impervious surface receiving flow control, the flow control BMP type(s), area of impervious surface being infiltrated via an infiltration BMP, andarea of impervious surface not receiving flow control.  If available, provide a map depicting drainage basin boundaries for TDAs and subbasins for individual outfalls within a TDA (if applicable), and BMP locations.  This information can be summarized in the following table for each TDA/outfall. Some of this information can be provided in the table below, and some written description(s) may be necessary:

| Existing TDA/ Outfall Number | Total Area (acres) | Total Impervious Surface Area (acres) | Area w/ Runoff Treatment (acres) | Runoff Treatment BMP Type(s) | Area w/ No Runoff Treatment (acres) | Area w/ Flow Control (acres) | Flow Control BMP Type(s) | Area to Infiltration BMP (acres) | Area w/ No Flow Treatment (acres) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Totals |  |  |  |  |  |  |  |  |  |

___________________________________________________________________________
____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

5.	Describe the nature of the existing stormwater conveyance (drainage) system (e.g., pipe, culvert, channel, ditch, swale, sheet flow), including the drainage distance from project right-of-way to project receiving waterbody.  If available, provide a map of the conveyance system depicting TDA/outfall subbasin boundaries.

___________________________________________________________________________
____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

6.  To facilitate stormwater modeling efforts, for each TDA, fill out the Pre-project section of Form A (Inputs for End-of-Pipe Calculation).

Proposed Stormwater Facilities (Proposed Project)

7.	How many TDAs have been identified in the project area, how many outfalls or discharge point(s) are located in each TDA, and what are the receiving waterbodies?

| Proposed TDA Number | Number of Discharge Points/Outfalls | Receiving Waterbody |
| --- | --- | --- |

8.	For each proposed TDA/outfall (subdivide TDAs if there are multiple outfalls), identify total TDA area, area of impervious surface, area of impervious surface receiving runoff treatment, the runoff BMP type(s),  area of impervious surface not receiving runoff treatment, area of impervious surface receiving flow control, the flow control BMP type(s), area of impervious surface being infiltrated via an infiltration BMP, and area of impervious surface not receiving flow control.  If available, provide a map depicting drainage basin boundaries for TDAs and subbasins for individual outfalls within a TDA (if applicable), and BMP locations.  This information can be summarized in the following table for each TDA/outfall. Some of this information can be provided in the table below, and some written description(s) may be necessary:

| Existing TDA/ Outfall Number | Total Area (acres) | Total Impervious Surface Area (acres) | Area w/ Runoff Treatment (acres) | Runoff Treatment BMP Type(s) | Area w/ No Runoff Treatment (acres) | Area w/ Flow Control (acres) | Flow Control BMP Type(s) | Area to Infiltration BMP (acres) | Area w/ No Flow Treatment (acres) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Totals |  |  |  |  |  |  |  |  |  |

___________________________________________________________________________
____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

9.	If no runoff treatment or flow control BMPs are proposed for a TDA, provide justification.

___________________________________________________________________________
____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

10.	Describe the nature of the proposed stormwater conveyance (drainage) system (e.g., pipe, culvert, channel, ditch, swale, sheet flow), including the drainage distance from project right-of-way to project receiving waterbody.  If available, provide a map of the conveyance system depicting TDA/outfall subbasin boundaries.

___________________________________________________________________________
____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

11. For each TDA, fill out the Proposed Project section of Form A (Inputs for End-of-Pipe Calculation).

12. Are any of the project’s proposed TDAs exempt from the flow control requirement per the most recent version of the Highway Runoff Manual?    Yes    No

If yes, identify the exempt TDA(s) and basis for exemption:

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

If no, and the project is petitioning for an exemption, has a hydrologic analysis supporting the exemption been approved by Ecology?    Yes    No

- If yes, provide a summary of the analysis as an attachment to this checklist.

- If no, a hydrologic analysis justifying the exemption must be submitted to Ecology for approval or flow control must be provided.

Note: For some receiving waterbodies, the project biologist may need to acquire more detailed information flow related impacts.

13.	For each non-exempt TDA, identify the total area of new impervious surface to receive flow control and for western Washington, the predeveloped land cover design standard (grass, pasture, or forested):

| TDA Number | Square Feet, Acres | Predeveloped Land Cover Design Standard (western Washington only) |
| --- | --- | --- |

14.	Will any existing impervious surface be retrofitted for flow control?   Yes    No

If yes, identify the total area of the existing impervious surface in each TDA will be retrofitted for flow control and for western Washington, the predeveloped land cover design standard (grass, pasture, or forested):

| TDA Number | Square Feet, Acres | Predeveloped Land Cover Design Standard (western Washington only) |
| --- | --- | --- |

15.	For western Washington, is the project able to provide all the required flow control for new impervious surfaces within the project limits to the historic land cover standard?   
 Yes    No

If no, identify where and how this project-triggered retrofit obligation will be met off-site, including the location(s) and the applicable land cover design standard (grass, pasture, or forested):

| On-Site/Off-Site Location TDA Numbers | Volumetric Difference Between Off-site and On-site Volume Detained | Land Cover Design Standard (western Washington only) |
| --- | --- | --- |

16.	Is the project able to provide all the required flow control for replaced impervious surfaces within the project limits?   Yes    No

If yes, for each TDA, identify total area of the replaced pollution-generating impervious surface to receive flow control and for western Washington, the predeveloped land cover design standard (grass, pasture, or forested):

| TDA Number | Square Feet, Acres | Predeveloped Land Cover Design Standard (western Washington only) |
| --- | --- | --- |

If no, identify where and how this project-triggered retrofit obligation will be met off-site, including the location(s) and the applicable land cover design standard (grass, pasture, or forested):

| Off-Site Location TDA Number | Square Feet, Acres | Land Cover Design Standard |
| --- | --- | --- |

17.	Does the project transfer water between watersheds?  Yes    No

18.	Will the project require construction of a new stormwater outfall structure or a new point 
of discharge to any water body?    Yes    No

If yes, identify the receiving water body and describe areas of permanent and temporary clearing or grading, types of vegetation to be removed, amount of riprap, diameter of outfall pipe(s), and all maintenance/access roads to be constructed.  If available, provide a map of outfall locations.

__________________________________________________________________________
__________________________________________________________________________
______________________________________________________________________________________________________________________________________________________________________________________________________________________________

19.	If the project is not infiltrating all of the runoff from the new impervious surface and is unable to provide the required runoff treatment or flow control for the entire new impervious surface, explain why not.  Documentation should include a completed copy of the Engineering and Economic Feasibility (EEF) Evaluation Checklist (Appendix 2A).

_______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

20.	What stormwater management design standards were applied?

  WSDOT Highway Runoff Manual, version _____________________________________
      (1995, 2004, 2006, 2007, 2008, etc.)

  Ecology’s Stormwater Management Manual(s), version ___________________________
      (2001, 2005 Western Washington; 2004 Eastern Washington, etc.)

  Other: __________________________________________________________________

  Not Applicable

21.	Will project require dilution modeling?   The project biologist will determine this for each project TDA by completing the End-of-Pipe loading analysis from HI-RUN Model,based upon the information provided in this checklist.	 Yes    No    Not Determined

If no, or not determined, this form is complete.

If yes, the project biologist will contact the project engineer/designer and request that Part 2 be filled out for each discharge point/outfall in the TDA(s) of interest:   If not determined, do not continue on to Part 2, until the project biologist(s) determine if it is necessary.

Prepared by_____________________________  Phone________________  Date____________

Project Engineer___________________________  Office Location_______________________

Part 1 (continued)

Form A – TDA ______

Inputs for HI-RUN Model End-of-Pipe Loading Subroutine

Baseline (i.e., Pre-Project) Stormwater Facilities

| Treatment Type | Level of Infiltrationa | Subbasin 1 Impervious Area (acres) | Subbasin 2 Impervious Area (acres) | Subbasin 3 Impervious Area (acres) | Subbasin 4 Impervious Area (acres) | Subbasin 5 Impervious Area (acres) |
| --- | --- | --- | --- | --- | --- | --- |
| □ Basic OR | 0% |  |  |  |  |  |
| □ Phosphorus | 20% |  |  |  |  |  |
| (Check one) | 40% |  |  |  |  |  |
|  | 60% |  |  |  |  |  |
|  | 80% |  |  |  |  |  |
| Enhanced | 0% |  |  |  |  |  |
|  | 20% |  |  |  |  |  |
|  | 40% |  |  |  |  |  |
|  | 60% |  |  |  |  |  |
|  | 80% |  |  |  |  |  |
| None |  |  |  |  |  |  |
| Infiltration BMP | 100% |  |  |  |  |  |

a Level of infiltration relates to the amount of incidental infiltration that can be expected, expressed as a percentage of annual average flow volume.  If no incidental infiltration can be assumed, enter area in the row corresponding to “0%”

Proposed (i.e., Post Project) Stormwater Facilities

| Treatment Type | Level of Infiltrationa | Subbasin 1 Impervious Area (acres) | Subbasin 2 Impervious Area (acres) | Subbasin 3 Impervious Area (acres) | Subbasin 4 Impervious Area (acres) | Subbasin 5 Impervious Area (acres) |
| --- | --- | --- | --- | --- | --- | --- |
| □ Basic OR | 0% |  |  |  |  |  |
| □ Phosphorus | 20% |  |  |  |  |  |
| (Check one) | 40% |  |  |  |  |  |
|  | 60% |  |  |  |  |  |
|  | 80% |  |  |  |  |  |
| Enhanced | 0% |  |  |  |  |  |
|  | 20% |  |  |  |  |  |
|  | 40% |  |  |  |  |  |
|  | 60% |  |  |  |  |  |
|  | 80% |  |  |  |  |  |
| None |  |  |  |  |  |  |
| Infiltration BMP | 100% |  |  |  |  |  |

a	Level of infiltration relates to the amount of incidental infiltration that can be expected, expressed as a percentage of annual average flow volume.  If no incidental infiltration can be assumed, enter area in the row corresponding to “0%”.

See HI-RUN Users Guide for instructions on completing these tables.

Part 2

Form B – TDA ____

Inputs for HI-RUN Model Receiving Water Dilution Subroutine

| Stormwater Parameter | Background Concentration (mg/L) |
| --- | --- |
| Total Suspended Solids |  |
| Copper – Total |  |
| Copper – Dissolved |  |
| Zinc – Total |  |
| Zinc – Dissolved |  |

Drainage Subbasin #_____

| Receiving Water Characteristics Downstream from Discharge | Month |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Jan. | Feb. | Mar. | Apr. | May | Jun. | Jul. | Aug. | Sept | Oct. | Nov. | Dec. |
| Stream depth (ft) |  |  |  |  |  |  |  |  |  |  |  |  |
| Stream velocity (fps) |  |  |  |  |  |  |  |  |  |  |  |  |
| Channel width (ft) |  |  |  |  |  |  |  |  |  |  |  |  |
| □ Stream slope (ft/ft) OR□ Manning’s roughness “n” (Check one) |  |  |  |  |  |  |  |  |  |  |  |  |
| Discharge distance into receiving waterbody from nearest shoreline |  |  |  |  |  |  |  |  |  |  |  |  |

Drainage Subbasin #_____

| Receiving Water Characteristics Downstream from Discharge | Month |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Jan. | Feb. | Mar. | Apr. | May | Jun. | Jul. | Aug. | Sept | Oct. | Nov. | Dec. |
| Stream depth (ft) |  |  |  |  |  |  |  |  |  |  |  |  |
| Stream velocity (fps) |  |  |  |  |  |  |  |  |  |  |  |  |
| Channel width (ft) |  |  |  |  |  |  |  |  |  |  |  |  |
| □ Stream slope (ft/ft) OR□ Manning’s roughness “n” (Check one) |  |  |  |  |  |  |  |  |  |  |  |  |
| Discharge distance into receiving waterbody from nearest shoreline |  |  |  |  |  |  |  |  |  |  |  |  |

See HI-RUN Users Guide for instructions on completing these tables